SAMPLE Lockout/Tagout Procedures for

Automated Palletizer Wrap Station

Machine Number: 2   Serial Number: 123456789

Date Reviewed 6/1/2021

In order to prevent unexpected startup or movement of parts that could cause an injury, this procedure will be used when performing maintenance or repairs to the listed equipment.

______________________________________________________________________________

Sources of hazardous energy:

Electrical: 220 volts		Fuel: ____		Spring: Y/N ____ 	Comp. Air: 120 psi

Flywheel: Y/N ___		Hydraulic: ___ pressure			Chemical: _______

Thermal: ____ temp 		Gravity affect? Y/N ___			Steam: Y/N _____

Jammed Equipment force? Y/N No List _____________________________________

- Authorized employees will be familiar with the types, sources and magnitude of hazardous energy, for this machine. The Energy Source Evaluation sheet documents the energy sources.

- The Authorized employee will notify affected employees that the machine will be shut down and locked out.

- The machine will be turned OFF or shut down using normal stopping procedures shown here:

Push red STOP button on front of palletizer.

- Isolate all energy sources listed above by using the following instructions: (throw identified and labeled switches.)

*Move #2 safety switch handle to the OFF position.

*Move valve handle #1 to the CLOSED position. This will be perpendicular to the air line. See picture. Bleed air from the palletizer by pressing the Bleed Air button.

- Apply pad lock thru the switch handle and box hole used in step 4.

Apply valve cover and secure pad lock thru the valve cover.

- Stored energy will be relieved, disconnected and rendered safe. Attempt to start the machine.

- Verify all isolation devices have been installed and that de-energization has occurred.

*If the employee who applies the LOTO devices must leave the workplace, follow the transfer steps listed in the LOTO policy.

_______________________________________________________________________________

Release from LOTO-Before Re-Starting Equipment

- Inspect the work area to ensure tools, parts, rags, and other non-essential items have been removed from the equipment and the area.

- Notify employee that the machine will soon be started, and they are to clear the area.

- The LOTO devices are to be removed by the employee who installed them. If the employee is not present, follow the required transfer steps in the LOTO policy.

- Start up.

This procedure will be reviewed for accuracy annually. It will also be used in annual retraining.